EEPROM – Electrically Erasable Programmable Read Only Memory

This is a user modified read only memory, it can be erased and reprogrammed (written to) repeatedly by applying a high voltage. But the data cannot be erased selectively, it has to be erased entirely by the electric voltage.

For modification of program in EEPROM it doesn’t have to be taken out of the computer or device.

Types of EEPROM:

- Serial EEPROM – data is transferred serially, so its slower than parallel EEPROMs. Serial EEPROM protocol is made up of three phases—OP-code phase(Read/Write), address phase, data phase.

- Parallel EEPROMs – data is transferred parallelly, and hence faster. It is much denser and reliable, but its cost is high and there is low popularity of parallel EEPROMs.

EEPROMs are used in many applications including telecom, consumer, automotive and industrial applications.

Most of the time when using an EEPROM we don’t have to see all the contents of the memory at once, but read and write bytes here and there as needed.

Reading and writing sequence in a typical EEPROM:

(Serial EEPROM AT25512)

CS: Chip Select, SI: Slave In, SO: Slave Out, RDSR: Read Status Register

READ—

- CS line is pulled low, read op-code is sent, address from which to read is sent—through SI line

- The data from that address is shifted on to the SO line.

- If only one byte is read, the CS line should be pulled high after transmission

- The byte address is automatically incremented, so the read sequence can be continued without pulling CS line high.

WRITE—

- In order to write, first the device has to be write enabled. This is done by sending the WREN instruction first.

- If an internal write cycle is ongoing, all commands will be ignored except for the RDSR instruction

- The CS line is pulled low, write op-code is sent via the SI line.

- This is followed by the byte address, and then the data to be written.

- The programming will start after the CS pin is brought high. To check whether the device is ready or busy, the RDSR instruction can be sent. If bit 0 is ‘1’ then write cycle is still in progress.

- After each byte of data is received the address bits are internally incremented, and if the data exceeds 128 bytes, then counter rolls over, and bytes are overwritten.

Page write Buffer

Most EEPROM devices have this ‘page write buffer’ for writing multiple bytes to the slave device the same way we write a single byte. There is an internal counter, which automatically increases the memory location with each data byte received.