Microcontrollers Basics

Microcontroller is a small sized computer (computer-on-a-chip) that can be embedded on a chip. Microcontroller or an MCU is used to control other parts of an operating system—usually with the use of a microprocessor, memory, and some peripherals.

Microcontrollers are embedded in some other device to run some program, and MCUs are dedicated to one task and run one specific program.

The main program is stored in the ROM(read only memory) and generally does not change. A microcontroller has a dedicated input device, and the output is usually through an LED or LCD display.

One of the most significant requirements of an MCU in an embedded system is to reduce power consumptions and space.

Elements of a Microcontroller:

- The central processing unit: performs arithmetic operations and controls data flow. It hides the complex circuitry and the CPU functionality is not visible to the user.

- Memory: The program instructions is store in a non-volatile memory that tell the CPU what to do. Volatile memory, like RAM is used to store temporary data and is usually lost when the system turns off.

Program counter increases by one each time the program instructions run, helps to keep track of the counter’s place in the line of code.

- Peripheral: These are hardware that helps a microcontroller to interact with the outside environment. Clock generation, phaselocked loops, timers, analog signal processing, I/O devices, Serial Communication etc are part of the peripherals

- Support circuits: Some circuits that cant be termed as peripherals, but they support the internal workings of the MCU – ex: Interrupts, debug circuits (tracks down buys and optimizes performance), power supply circuitry, clock generation modules etc

Phase Locked Loop

It is an electronic circuit with an oscillator which constantly adjusts to match the frequency of an input signal. They consist of a voltage-controlled oscillator—the VCO seeks and locks onto the desired frequency. When there is a difference between a VCO frequency and the reference frequency, the phase comparator produces an error.